Practical Robotics Projects with Arduino

(CSE 4571)

Lab Assignment No – 04

ULTRASONIC & IR SENSING

Submission Date: _______________

| Branch: CSE Section: |  |  |
| --- | --- | --- |
| Name | Registration No. | Signature |

Department of Computer Science and Engineering

Institute of Technical Education and Research (Faculty of Engineering)

Siksha 'O' Anusandhan (Deemed to be University)

Bhubaneswar, Odisha-751030.

# Aim:

# Ultrasonic & IR Sensing – To Interface HC-SR04 and IR sensors with Arduino UNO for distance measurement and obstacle detection.

# Objectives:

- To study the working principle of Ultrasonic (HC-SR04) and Infrared (IR) sensors.

- To interface HC-SR04 ultrasonic sensor with Arduino UNO for distance measurement and display the output values on I2C LCD.

- To interface IR sensor with Arduino Uno for obstacle detection using buzzer and LED.

- To validate distance measurement of HC-SR04 ultrasonic sensor using LEDs as indicators.

# Pre-Lab Questionnaire:

A. Experiment-Specific

- What is the working principle of the HC-SR04 ultrasonic sensor for distance measurement?

- Mention the function of Trigger pin and Echo pin in the HC-SR04 sensor.

- Write the formula to calculate distance from the ultrasonic sensor using the speed of sound.

- What is the typical range of distance measured by an HC-SR04 ultrasonic sensor?

- What is the working principle of an Infrared (IR) sensor used for obstacle detection?

- Differentiate between active IR sensor and passive IR sensor with examples.

- State any two applications of ultrasonic sensors in real life.

- Why is it necessary to use pinMode() and digitalWrite() functions in Arduino while interfacing sensors?

- How does an IR sensor distinguish between the presence and absence of an obstacle?

- Mention one key advantage and one limitation of ultrasonic sensors compared to IR sensors?

Answers to Pre-Lab Questions

Components/Equipment Required:

| Sl. No. | Name of the Component / Equipment | Specification | Quantity |
| --- | --- | --- | --- |
| 1) | Arduino UNO R3 | 16MHz | 1 |
| 2) | Arduino UNO cable | USB Type A to Micro-B | 1 |
| 3) | Ultrasonic sensor HC-SR04 |  | 1 |
| 4) | IR sensor module | Obstacle detection | 1 |
| 5) | I2C LCD |  | 1 |
| 6) | Buzzer |  | 1 |
| 7) | Resistors (carbon type) | 220Ω / 330Ω | 2 |
| 8) | LED | Any colour of your choice | 8 |
| 9) | Breadboard | 840 Tie points | 1 |
| 10) | Jumper Wire | --------------------------- | As per requirement |

Objective 2

To interface HC-SR04 ultrasonic sensor with Arduino UNO for distance measurement and display the output values on I2C LCD.

## Circuit / Schematic Diagram

Figure 1: HC-SR04 Ultrasonic Sensor Pinout

Figure 2: Work of HC-SR04

Figure 3: Ultrasonic Distance measurement circuit using HC-SR04 & Arduino Uno

Figure 4: Ultrasonic Distance Measurement Output in LCD Display

## Code

Write an Arduino program to interface HC-SR04 ultrasonic sensor with Arduino Uno for distance measurement and display the output value in I2C LCD.

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#define trigPin 9

#define echoPin 10

LiquidCrystal_I2C lcd(0x27, 16, 2);  // I2C address 0x27, adjust if needed

void setup() {

lcd.init();

lcd.backlight();

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

lcd.setCursor(0, 0);

lcd.print("Distance Meter");

delay(1000);

lcd.clear();

}

void loop() {

long duration;

float distance;

// Send a 10µs pulse to trigger pin

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read echo pin and calculate distance

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.0343 / 2; // cm

// Handle out-of-range

if (distance > 400 || distance < 2) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Out of range");

} else {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Distance:");

lcd.setCursor(0, 1);

lcd.print(distance, 1);

lcd.print(" cm");

}

delay(500); }

## Observation

Figure 5: (Simuation based distance measurement using HC-SR04 ultrasonic sensor and output display in I2C LCD)

Figure 6: (Hardware Implementation based distance measurement using HC-SR04 ultrasonic sensor and output display in I2C LCD)

# Objective 3

To interface IR sensor with Arduino Uno for obstacle detection using buzzer and LED.

Figure 7: Interface IR sensor with Arduino UNO for obstacle detection

## Code

Write an Arduino program to detect an obstacle in front, and turn ON the LED and buzzer.

#define TRIG_PIN    2

#define ECHO_PIN    3

#define LED_PIN     5

#define BUZZER_PIN  4

long duration;

int distance;

void setup() {

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

Serial.begin(9600);

}

void loop() {

// Clear trigger

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Send 10 µs pulse

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read echo

duration = pulseIn(ECHO_PIN, HIGH);

// Convert to cm

distance = duration * 0.034 / 2;

Serial.print("Duration: ");

Serial.print(duration);

Serial.print("\tDistance: ");

Serial.print(distance);

Serial.println(" cm");

// Buzzer and led activation when distance < 30

if (distance > 0 && distance <= 30) {

digitalWrite(LED_PIN, HIGH);

digitalWrite(BUZZER_PIN, HIGH);

} else {

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

}

delay(500);

}

Observation

Figure 8: (Simulation based interfacing IR sensor and buzzer with Arduino UNO for obstacle detection)

Figure 9: (Hardware Implementation of interfacing IR sensor and buzzer with Arduino UNO for obstacle detection)

Objective 4

To validate distance measurement of HC-SR04 ultrasonic sensor using LEDs as indicators.

## Circuit / Schematic Diagram

Figure 10: HC-SR04 + Arduino Uno +LCD+LED

Code

Write an Arduino program to integrate an ultrasonic sensor, an I2C LCD display, and LED indicators for real-time distance measurement and visualization.

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#define TRIG_PIN 3

#define ECHO_PIN 2

// LEDs from pins 4–13

int leds[] = {4, 5, 6, 7, 8, 9, 10, 11, 12, 13};

const int ledCount = 10;

// Initialize I2C LCD (usually address 0x27 or 0x3F)

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(9600);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

// setup all LEDs

for (int i = 0; i < ledCount; i++) {

pinMode(leds[i], OUTPUT);

}

// initialize LCD

lcd.init();

lcd.backlight();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Distance Sensor");

delay(1000);

}

void loop() {

long duration;

float distance;

// Trigger pulse

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read echo pulse duration

duration = pulseIn(ECHO_PIN, HIGH, 30000);

// Convert to cm

distance = duration * 0.034 / 2;

// Safety check: ignore invalid readings

if (distance == 0 || distance > 400) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Out of range   ");

Serial.println("Out of range");

} else {

// Print distance

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Distance:");

lcd.setCursor(10, 0);

lcd.print(distance, 1);

lcd.print("cm");

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

}

// LED visualization (equal gaps from 4 cm to 320 cm)

int numLEDs = map(distance, 4, 320, 0, ledCount);

numLEDs = constrain(numLEDs, 0, ledCount);

for (int i = 0; i < ledCount; i++) {

if (i < numLEDs) {

digitalWrite(leds[i], HIGH);

} else {

digitalWrite(leds[i], LOW);

}

}

delay(300);

}

## Observation

Figure 11: (Software Implementation of interfacing ultrasonic sensor, an I2C LCD display, and LED indicators)

Figure 12: (Hardware Implementation of interfacing ultrasonic sensor, an I2C LCD display, and LED indicators)

Conclusion:

Precautions:

Post Experiment Questionnaire:

- Experiment-Specific (LED Blinking & LED Patterns)

- What happens if the Trigger pin of the HC-SR04 ultrasonic sensor is not given a proper 10 µs pulse?

- If the Echo pulse duration is measured as 2 ms, calculate the distance of the obstacle. (Speed of sound = 343 m/s)

- Why do ultrasonic sensors measure distance more accurately than IR sensors in outdoor conditions?

- What would be the effect of bright sunlight on the IR sensor’s performance?

- How can you modify the Arduino code to turn ON an LED when an obstacle is detected within 15 cm using the HC-SR04 sensor?

- If the ultrasonic sensor reads 200 cm, what should be the approximate Echo pulse duration?

- Why is it necessary to divide the total sound travel time by 2 in ultrasonic distance measurement?

- What logic output does the IR obstacle sensor provide when an object is detected?

- Mention one limitation of using only IR sensors for obstacle detection in mobile robots.

- Suggest a practical application where both ultrasonic and IR sensors are used together for better performance.?

| (Signature of the Faculty) | (Signature of the Student) |
| --- | --- |
|  | Name: |
| Date: | Registration No.: |
|  | Branch: |
|  | Section |